| MATERIAL LIST |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Project Name |  |  |  |  |  |  |  |
| 85.72m2 Light steel Villa |  | Unit Price |  | Quantity (Units) | Total Building Area(m2) | Total price (USD) |  |
|  |  | FOB Tianjin |  | 1 | 85.72 | US$34,510.00 |  |
| Material List |  |  |  |  |  |  |  |
| NO. | Items | Specification |  | Units | Quantity |  |  |
| 1 | Main structure | Galvanized Light steel structure |  | m2 | 85.72 |  |  |
| 2 | Exterior Wall | coating+waterproof putty |  | m2 | 127.94 |  |  |
|  |  | 12mm gypsum board |  |  |  |  |  |
|  |  | 75mm,16K/m3 glass wool |  |  |  |  |  |
|  |  | 10mm calcium silicate board |  |  |  |  |  |
|  |  | waterproof paper |  |  |  |  |  |
|  |  | ventilated steel keel |  |  |  |  |  |

| Material List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | Items | Specification |  | Units | Quantity | Price/m2 | Sub-total price |
|  |  | WPC cladding panel/aluminous gusset plate |  |  |  |  |  |
| 3 | Interior wall of normal rooms | coating+waterproof putty |  | m2 | 22.43 |  |  |
|  |  | 2 layers of 12mm gypsum board |  |  |  |  |  |
|  |  | structure steel stud |  |  |  |  |  |
|  |  | 75mm,16K/m3 glass wool |  |  |  |  |  |
|  |  | 2 layers of 12mm gypsum board |  |  |  |  |  |
|  |  | waterproof putty+coating |  |  |  |  |  |
| 4 | Interior wall of kitchen/bathroo | structure steel stud |  | m2 | 97.77 |  |  |
|  |  | 75mm,16K/m3 glass wool |  |  |  |  |  |
|  |  | calcium silicate board |  |  |  |  |  |

| Material List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | Items | Specification |  | Units | Quantity |  |  |
|  | m | steel mesh |  |  |  |  |  |
|  |  | JS waterproof layer(coating height 1800/300mm) |  |  |  |  |  |
|  |  | glazed tile |  |  |  |  |  |
| 5 | plane roof | 3mm SBS waterproof roll |  | m2 | 99.89 |  |  |
|  |  | 18mm structure board |  |  |  |  |  |
|  | Pitched roof | structure board |  |  |  |  |  |
|  |  | 3mm SBS waterproof roll |  |  |  |  |  |
|  |  | 20*30 counter batten+20*30 roof batten |  |  |  |  |  |
|  |  | stone-coated metal roofing tile |  |  |  |  |  |
| 6 | Parapet | inside：steel struture+10mm calcium silicate board+3mm SBS waterproof roll+75mm/16K glass wool+10mm calcium silicate board+waterproof paper+ventilated steel keel+WPC cladding panel | Pls refer to the above related pictures | m2 | 16.48 |  |  |
| 7 | Cornice | steel struture+10mm calcium silicate board+waterproof putty+coating | Pls refer to the above related pictures | m2 | 15.90 |  |  |

| Material List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | Items | Specification |  | Units | Quantity |  |  |
| 8 | Gutter | aluminium alloy |  | m | 62.40 |  |  |
| 9 | Floor of normal rooms | floor tile |  | m2 | 62.51 |  |  |
|  |  | waterproof plastic film+10mm composite wood floor |  |  |  |  |  |
|  |  | 5mm foamed plastic cushion+waterproof plastic film |  |  |  |  |  |
| 10 | Floor of kitchen/bathroo m | non-slippery tile |  | m2 | 23.21 |  |  |
|  |  | JS painting waterproof layer |  |  |  |  |  |
|  |  | waterproof plastic film |  |  |  |  |  |
|  |  | light steel ceiling |  |  |  |  |  |
|  |  | 75mm,16K/m3 glass wool |  |  |  |  |  |

| Material List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | Items | Specification |  | Units | Quantity |  |  |
| 11 | Ceiling of normal rooms | waterproof paper |  | m2 | 62.51 |  |  |
|  |  | 9.5mm gypsum board |  |  |  |  |  |
|  |  | waterproof putty+coating |  |  |  |  |  |
| 12 | Ceiling of kitchen/bathroo m | light steel ceiling+75mm/16K glass wool+moistureproof film | Pls refer to the above related pictures | m2 | 23.21 |  |  |
|  |  | PVC panel |  |  |  |  |  |
| 13 | Outside door | sliding door |  | m2 | 12.60 |  |  |
| 14 | Outdoor steps and floor | non-slippery tile |  | m2 | 41.90 |  |  |
| 15 | Inside door | paint-free door |  | m2 | 9.03 |  |  |
| 16 | Window | single-glass plastic steel window |  | m2 | 11.28 |  |  |
| 17 | Windowsill | manu-marble |  | m | 6.90 |  |  |

| Material List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | Items | Specification |  | Units | Quantity |  |  |
| 18 | Sanitary appliance | toilet |  | m2 | 85.72 |  |  |
|  |  | shower |  |  |  |  |  |
|  |  | basin with pedestal |  |  |  |  |  |
| 19 | Water supply and drainage | indoor water pipe+valve+watermiter |  | m2 | 85.72 |  |  |
|  |  | water heater |  |  |  |  |  |
| 20 | Electrical system | swith+faceplate+line pipe+lamp+distribution box |  | m2 | 85.72 |  |  |
| 21 | Lightning- protection system |  |  | m2 | 85.72 |  |  |
| 22 | Light-current system | for TV,telephone,network |  | m2 | 85.72 |  |  |
| 23 | Packing charge |  |  | m2 | 85.72 |  |  |
| 24 | shipment Load |  |  |  |  |  |  |
| 25 | Total price of materails |  |  |  |  |  | US$34,510.00 |

| Material List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | Items | Specification |  | Units | Quantity |  |  |
| Note: - This quotation is based on the design drawings, including the cost of the main building materials above the foundation; 2.This quotation doesn't include the following: - outdoor plinth materials; - materials used for foundation, like cement,sand,reinforced bar,stone,mortar, auxiliary materials of concrete,additive,admixture,glue etc.; (3)charge of installation guide in the construction site. - The quotation should be adjusted accordingly to actual situation if necessary. - The material pictures is just for reference, more options will be supplied before the production. |  |  |  |  |  |  |  |