| SPEC G2 VEHICLE SPEC |  |  |  |
| --- | --- | --- | --- |
| Product Name |  | Terriitori G2 Passenger Electrical Vehicle JEEP |  |  |
| Item |  | Specification | Reference Pictures |  |
| Overall Size |  | 3300×1600mm×1700mm |  |  |
| steering wheel |  | LHD(Left side steering wheel)&RHD |  |  |
| Seats Capacity |  | 4 Seats |  |  |
| Curb Weight |  | 645Kg(with battery) |  |  |
| Engine Power |  | 60V4000W Motor 60V 4000W |  |  |
| Battery |  | 60V 103AH, Lead acid Battery Recommend |  |  |
| Controller |  | Special Controller |  |  |
| Charger Type |  | Vehicle-mounted |  |  |
| Driving Mode |  | Rear Mounted / Rear Drive |  |  |
| Damping system F |  | Integrated Front Axle + longitudinal arm torsion beam Non-independent suspension |  |  |
| Damping system R |  | Integrated Axle + longitudinal arm torsion beam Non-independent suspension |  |  |
| Car Body Structure |  | Frame type+ Stamping |  |  |
| Display Instrument |  | Liquid Crystal Electronic Digital Display 7 Inch +MP5 |  |  |
| Brake System |  | F Disc / R Disc |  |  |
| Brake Type |  | Pedal Oil Brake |  |  |
| Tyre |  | 165/70R 12 Vacuum Tires |  |  |
| Wheel Hub |  | Iron hub + cover |  |  |
| Maximum Speed |  | 50 Km/H |  |  |
| Charging type& Time |  | China type, 8~10Hours (General) |  |  |
| Endurance Mileage |  | about 100-120km |  |  |
| Driving&Copilot Seats |  | Manual Adjusted Seats, Two Directions |  |  |
| Headrest |  | not adjustable |  |  |
| Central Lock |  | Yes with Remote key |  |  |
| Intelligent Remote Key |  | Yes |  |  |
| Cooling System |  | with air conditioner |  |  |
| Security System |  | Backup Camera +MP5 |  |  |
| Entertainment System |  | MP5 Multimedia Player, Stereo Speaker MP5 |  |  |
| Light Combination |  | ①LED Headlamp,LED ②Standalone Steering Lamp, ③Taillight,④ High-mount Stop Lamp, |  |  |
| Other Configuration |  | ①Windshield Wiper, ②Power Windows, ③Folding & Adjustable Rearview Mirror |  |  |
| Color options |  | ①White ② Red ③Blue ④Pink |  |  |
| Container Load |  | CBU 4 Sets in 40HQ |  |  |
| Shipment Port |  | Qingdao/Shanghai/Lianyungang Port |  |  |
| Remarks |  | *1. About this Quotation, Barring special circumstances, the above quotations valid in 15 workdays. *2. About Packaging, Standard export packing, or customized packing as your request *3. About Payment, By T/T: 50% amount in advance, 50% amount paid before delivery. *4. About Delivery time, Based on actual quantity, 25 working days in general after payment receipt confirmed. *5. About Quality Guarantee,12 months for Main Parts, include motor/controller/frame/battery. |  |  |

( Please sign back when confirmed )